10.12.2019

Tomi Korkalainen, Aleksa Lintula, Jani Koivula Juuso Saari, Sebastian Mäkipää

This work is licensed under the Creative Commons Attribution-ShareAlike 4.0 International License. To view a copy of this license, visit

https://creativecommons.org/licenses/by-sa/4.0/

Autodesk Fusion 360

Tutorials

General Information

| Overview |  | This document unfolds the basic functions of Autodesk Fusion 360 and goes through three small tutorials. They will include the essential tools for beginners for them to have a quick start on it. |
| --- | --- | --- |
| Project management |  | Fusion Team is a centralized source for all your project collaboration. Projects and designs are always created within a team. Before you can start modelling, you need to create a team if you have not already. Active team can be changed from the top right corner. Team can be switched from the top right corner Projects can and often will consist of multiple designs. Projects are the intended way of keeping the related designs organized in the same place. You can share projects with other Fusion 360 users by being in the same team and giving the other person privileges to the project. |
| Data panel |  | Open the Data Panel to see your team’s projects, create new and open existing ones. The Data Panel button can be found from the upper left corner of the program. The active team can be switched under your username from the upper right corner. Preferences are also found here. Data Panel button location |
| File format |  | Export and other basic functions can be found under the file menu. File Menu button can be found next to the Data Panel button. For the design to be compatible with most of the 3D printing software you should export the file as STL-format. File button location |
| Timeline |  | You can move the timeline marker to return to different points of the project. Each step is shown in the timeline and can be distinguished by the tool icons. The steps can be edited from the timeline by right clicking the step and clicking Edit Feature. Pressing the play button showcases every step of the project like a slideshow. Timeline / History |
| Hotkeys |  | Some tools can be hard to find from the menus, so learning the hotkeys for the basic tools can be useful. Keyboard sticker sheet |

Cup – Tutorial

| Overview |  | This tutorial contains instructions how to 3D-model a simple cup. Tools included in this tutorial: - Plane - Sketch - Extrude - Fillet - Shell; 3D modelled cup. |
| --- | --- | --- |
| Sketch |  | Before making a sketch to the project, a plane must be selected. Planes can be thought as working surfaces. Created sketches will be placed on the selected plane, so they will be on the same face. At start of each design you are given 3 planes to work with. Plane Selection and the View cube Click on the Create Sketch button. The button can be found in the toolbar at the top of the view, in the solid tab. Now, select the XZ Plane by clicking it. If you find any trouble selecting the correct plane, press the top face of the View Cube. The View Cube is located on the upper right corner of the view. After pressing the top face, you should only see the XZ Plane. After the plane selection, sketch tab and sketch palette will appear on the toolbar. Select Center Diameter Circle tool. The tool can be found in the toolbar at the top of the view, in the sketch tab. With this tool, a circle can be drawn by placing two points, one on the center and one on the circumference. Click to place the center point and specify diameter of the circle with mouse or by entering a value. After entering the value, press Finish Sketch from the Sketch Palette menu. The circle sketch has now been finished. Finished Circle Sketch |
| Extrude |  | After creating the sketch, it can be extruded into a 3D object. Select the Extrude tool to create a cylinder out of the circle sketch. The tool can be found in the toolbar at the top of the view, in the solid tab. Click on the circle sketch that was drawn during the previous step. To adjust the extrude distance, drag the blue arrow or enter a value to the distance field in the extrude menu. Then press OK from the Extrude menu. Circle extruded 70mm |
| Fillet |  | Select the Fillet tool. The tool can be found in the toolbar at the top of the view, in the solid tab. When the tool is activated, select the bottom edge of the body. Any edges will be highlighted when the mouse hovers over it. After an edge is selected, drag the blue arrow or enter the value to the radius field to add a fillet to the object. Then press OK from the Fillet menu. 15mm Fillet added to the bottom of the cup |
| Shell |  | Select the Shell tool. The tool can be found in the toolbar at the top of the view, in the solid tab. When the tool is activated, select the top face (flat surface) of the object. Adjust the thickness of the cup’s walls by entering a value on the shell menu or by dragging the blue arrow. Now press OK from the Shell menu. HINT - Fillet can also be added on the upper edges to make them smoother. Finished cup after shell and fillet on the top side corners |

Doorknob – Tutorial

| Overview |  | This is a step by step tutorial, on how to 3D model a doorknob. Tools included in this tutorial: - Plane - Sketch - Line - Conic Curve - 3-Point Arc - Sketch Dimension - Revolve - Hole - Circular Pattern; 3D modelled Doorknob. |
| --- | --- | --- |
| Plane |  | Select front view from the view cube and click Create Sketch icon and then select YZ Plane or XY Plane for the sketch. Default Planes and the View Cube |
| Lines |  | The sketch for the cross-section of a doorknob can be drawn easily with couple of lines. This tutorial demonstrates the use of three of the line tools: Line , 3-Point Arc and Conic Curve . Example photo of the sketch is located underneath. First draw straight horizontal and vertical lines using Line tool. Then draw the other lines using Conic Curves and 3-Point Arcs. Conic Curves are drawn by selecting three points. First two points determines start and end points for the line. Then place vertex point to modify the curve for the line. 3-Point Arc works essentially like the Conic Curve but maintains a circular shape. Select beginning, end point and then adjust arc. The result after drawing the lines |
| Sketch dimension |  | With the Sketch Dimension tool, points can be measured and locked to a specific length. Use it to lock vertical and horizontal line lengths. In the example photo below, vertical and horizontal lines have been locked with the dimension tool and their length is visible and can be changed. Modifying something in the project will not affect the length of these lines anymore. Finish the sketch by pressing OK from the sketch palette menu. Dimension tool used between horizontal points and vertical points |
| Revolve |  | Revolve the sketch into a 3D object using Revolve tool. The tool can be found in the toolbar at the top of the view, in the solid tab. Then select the sketch made in the previous step as the profile. Select the long vertical line as the axis. Check the picture below for reference. The sketch should be revolved around this line After selecting the axis, press OK from the Revolve menu. The sketch extruded into a 3D object |
| Hole |  | This step instructs on how to make a screw hole to the doorknob. Select bottom view from the View Cube. Then select Hole tool. It can be found in the toolbar at the top of the view, in the solid tab. View Cube Click on the bottom of the body to make a hole. Adjust the hole size, location, and finally press OK from the Hole menu. A hole added to the doorknob |
| Circularn Pattern |  | The sketch that you have created might be hidden, so make sure it is visible. It can be made visible by clicking the eye icon from the Browser. The Browser Select Circular Pattern tool. Select the previously made hole as the Object in the circular pattern menu and make sure that the pattern type is set as Faces as the created hole is not an object, but merely a face. Faces could be described as object’s surfaces in 3D modelling. So, we are basically copying the hole surface appear in multiple positions. Finally select the long vertical line from the sketch as the axis. The line that should be select as the axis After selecting the axis, new fields appear on the circular pattern menu. Enter quantity of the screw holes and press OK from the Circular Pattern menu. Finished doorknob with screw holes. |

Wall mounted coat hanger – Tutorial

| Overview |  | This tutorial gives a step by step guideline, on how to make a wall mounted coat stand. Tools included in this tutorial: - 2-Point Rectangle - Construction Line - Center Diameter Circle - Rectangular Pattern - Extrude; 3D modelled Wall Mounted Coat Stand. |
| --- | --- | --- |
| Sketch |  | First select the Sketch tool and click on one of the planes. Use the 2-Point Rectangle tool to draw the base of the coat stand. Press OK from the 2-Point Rectangle menu. Finished rectangle sketch |
| Construction Line |  | Select the Line tool. When the line tool is active, the sketch palette opens on the right. Under the sketch palette’s options, enable the Construction tool. Draw a construction line diagonally across the rectangle as shown in the picture below. Construction line across the rectangle |
| Center Diameter Circle |  | Draw a Center Diameter Circle in the middle of the rectangle. The cursor will be on the center when the triangle indicator appears. Click there to set the circle center and then finish it by giving it a radius. Circle drawn in the middle of the rectangle |
| rECTANGULAR PATTERN |  | Go to the sketch tab and select Rectangular Pattern tool. While the rectangular pattern tool is active, select the edge of the circle. Drag the arrow to duplicate the circle or enter a value to the distance field in the Rectangle Pattern menu. The quantity of the circles can also be changed on the rectangular pattern menu. Change direction type to symmetric to duplicate on both left and right sides of the initial circle. Now the sketch is finished, press OK from the Sketch Palette menu. Circle duplicated with Pattern tool |
| extrude |  | Select the Extrude tool. Now that the extrude tool is active, select all the circles and enter a value to the distance field in the extrude menu and press OK from the extrude menu. After extruding the circles, extrude the base of the hanger. The sketch might not be visible, so make sure that eye icon is active in the Browser menu. You can change the visibility by clicking the eye icon. Browser menu Finished coat hanger |